Simulation-Based Design and Verification of Finite State Machine (FSM) using ModelSim

Avani Agashe

Design.v

`timescale 1ns / 1ps

module fsm_detector (

input wire clk,       // Clock signal

input wire rst,       // Synchronous Reset

input wire din,       // Serial Data Input

output reg dout       // Output (Goes High '1' when "101" is detected)

);

// State Encoding using 2-bit local parameters

reg [1:0] current_state, next_state;

localparam IDLE = 2'b00,

S1   = 2'b01,

S10  = 2'b10;

// ----- Block 1: Sequential State Register -----

always @(posedge clk) begin

if (rst) begin

current_state <= IDLE;

end else begin

current_state <= next_state;

end

end

// ----- Block 2: Combinational Next-State Logic -----

always @(*) begin

case (current_state)

IDLE: begin

if (din == 1'b1) next_state = S1;

else             next_state = IDLE;

end

S1: begin

if (din == 1'b0) next_state = S10;

else             next_state = S1; // Stay here if it's another 1

end

S10: begin

if (din == 1'b1) next_state = S1;  // Pattern "101" completed, overlaps with next sequence

else             next_state = IDLE;

end

default: next_state = IDLE;

endcase

end

// ----- Block 3: Combinational Output Logic (Mealy Style) -----

always @(*) begin

case (current_state)

S10: begin

if (din == 1'b1) dout = 1'b1; // Output goes high the instant the final '1' arrives

else             dout = 1'b0;

end

default: dout = 1'b0;

endcase

end

endmodule

Testbench.v

`timescale 1ns / 1ps

module fsm_detector_tb;

// Inputs to the FSM (Declared as registers)

reg clk;

reg rst;

reg din;

// Outputs from the FSM (Declared as wires)

wire dout;

// Instantiate the Device Under Test (DUT)

fsm_detector uut (

.clk(clk),

.rst(rst),

.din(din),

.dout(dout)

);

// Generate Clock: Toggles every 5ns (10ns total clock cycle period)

always #5 clk = ~clk;

// Stimulus block

initial begin

// Step 1: Initialize signals and apply Reset

clk = 0;

rst = 1;

din = 0;

#15; // Hold reset through the first rising edge

rst = 0; // Release reset

// Step 2: Feed serial stream into the FSM on each clock period (10ns intervals)

// Testing sequence: 1 -> 0 -> 1 (First detection)

#10 din = 1;

#10 din = 0;

#10 din = 1; // At this moment, dout should go HIGH!

// Testing overlapping sequence: -> 1 -> 0 -> 1 (Second detection)

#10 din = 1;

#10 din = 0;

#10 din = 1; // At this moment, dout should go HIGH again!

// Feed a non-matching bit to return to IDLE

#10 din = 0;

#10 din = 0;

#20;

$finish; // End simulation

end

// Monitor text output in ModelSim Transcript panel

initial begin

$display("====================================================");

$display("         Starting Mealy FSM Simulation              ");

$display("====================================================");

$monitor("Time = %3dns | State = %b | Input = %b | Output = %b",

$time, uut.current_state, din, dout);

end

endmodule